SYSTEM REQUIREMENTS
SPECIFICATION

Industrial Dual-Motor Controller & Communication Gateway

| Document ID | IDMCG-SRS-001 |
| --- | --- |
| Date | 22 August 2026 |
| Project | Industrial Dual-Motor Controller |
| Version | V0.1 |
| MCU family | STM32 |
| Authority | System-level product requirements |

Important design rule: This document defines what the product shall do. It intentionally does not prescribe most implementation choices. The hardware/firmware designer is responsible for selecting regulators, motor-driver topology, MOSFETs, PHY/transceivers, connector series, PCB stackup, PDN strategy, filtering, thermal solution and component values, provided all requirements and verification criteria are met.

## System Requirements — v0.1

### 1. Product Definition

The product shall be a 24 VDC industrial controller designed to independently control two permanent-magnet DC motors and provide communication with external industrial control systems.

### 2. Motor Requirements

- Motor quantity: 2

- Motor type: Permanent Magnet Brushed DC Motor

- Motor model: 90ZYTA144-1253

- Motor nominal voltage: 12 VDC

- Motor nominal current: 3 A

- Each motor shall be independently controlled.

- The controller shall provide:

  - Enable/Disable

  - Direction control

  - PWM speed control

  - Current monitoring

  - Fault detection

- Peak/stall current shall be determined during detailed design and verified before final hardware release.

### 3. Main Power Requirements

- Nominal input voltage: 24 VDC

- Input voltage range: 20–30 VDC

- The controller shall generate all required internal power rails from the 24 VDC input.

- Input power protection shall be provided.

- The power architecture shall meet required voltage, current, efficiency, thermal and EMC requirements.

Implementation of power conversion is not fixed.
The designer may select LDO, switching regulator, isolated/non-isolated converter, filtering and protection components based on technical, thermal, EMC, availability and cost requirements.

### 4. Communication Interfaces

The controller shall provide:

CAN

- CAN interface according to applicable ISO 11898 requirements.

- Configurable communication bitrate.

RS-485

- RS-485 physical interface.

- Differential communication.

- Application protocol shall be defined separately; Modbus RTU may be supported.

Ethernet

- 10/100 Mbps Ethernet.

- Full-duplex operation.

- Ethernet PHY/interface shall be selected by the designer.

USB

- USB 2.0 Full-Speed

- Up to 12 Mbps.

- Intended for configuration, diagnostics, firmware update and service.

### 5. Internal Interfaces

The MCU shall provide interfaces required for internal peripherals, including:

- I²C

- SPI

- UART

- GPIO

- ADC

- PWM

- Timers

- DMA

- SWD/JTAG or equivalent debug/programming interface

The exact STM32 device shall be selected by the designer based on system requirements.

### 6. MCU Requirements

- MCU family: STM32

- MCU shall provide sufficient:

  - GPIO

  - ADC channels

  - PWM/timers

  - CAN capability/interface

  - UART

  - SPI

  - I²C

  - USB capability

  - Memory

  - Processing performance

- Exact STM32 part number is a design decision.

### 7. Monitoring

The controller shall monitor, where applicable:

- Input voltage

- Logic supply voltage

- Motor current

- Motor-driver fault status

- Board temperature

- Motor-driver temperature

- Communication status

### 8. Protection

The system shall provide appropriate protection against:

- Reverse polarity

- Over-voltage

- Under-voltage

- Over-current

- Short circuit

- Thermal overload

- Input transients

- ESD

- Electrical disturbances relevant to the intended industrial environment

The exact protection circuit and component selection are the responsibility of the designer.

### 9. Fault Handling

The controller shall:

- Detect critical faults.

- Disable affected motor output when required.

- Report fault status through the communication interfaces.

- Provide local fault indication.

- Prevent unsafe motor operation following defined fault conditions.

- Support fault recovery/reset according to firmware requirements.

### 10. Firmware

Firmware shall provide:

- Hardware initialization

- Motor control

- PWM generation

- Current/voltage/temperature monitoring

- Communication handling

- Fault management

- Diagnostics

- Configuration management

- Watchdog supervision

- Firmware update capability

Firmware shall be modular and maintainable.

### 11. PCB / Hardware Design

The PCB shall be designed for industrial operation and shall consider:

- Power integrity

- Signal integrity

- PDN

- Thermal management

- Current density

- Creepage/clearance where applicable

- EMC/EMI

- Grounding

- Decoupling

- Return-current paths

- Connector robustness

- Manufacturing constraints

PCB stack up, layer count, PDN architecture, decoupling strategy, regulator topology, connector type and individual component selection are design decisions.

### 12. Thermal

The designer shall evaluate:

- Power dissipation

- Motor-driver losses

- Regulator losses

- MCU and PHY losses

- PCB thermal performance

- Component temperature

- Derating

Thermal simulation and/or calculation shall be performed for critical components.

### 13. Environmental

Initial design target:

- Industrial indoor environment

- Operating temperature: TBD (To Be Determined)

- Storage temperature: TBD (To Be Determined)

- Relative humidity: TBD (To Be Determined)

- Vibration/shock requirements: TBD (To Be Determined)

These values shall be finalized before design freeze.

### 14. Verification

The final prototype shall be verified for:

- Power-up behavior

- Motor operation

- Motor current

- Communication

- USB

- CAN

- RS-485

- Ethernet

- Protection functions

- Fault handling

- Thermal performance

- Long-duration operation

- EMC pre-compliance

### 15. Standards / Design References

The design shall consider applicable requirements from:

- ISO 11898 — CAN

- TIA/EIA-485 — RS-485

- IEEE 802.3 — Ethernet

- USB 2.0 specification

- IEC 61000 series — EMC/environmental immunity and emissions as applicable

- Relevant industrial motor-drive/electrical safety standards where applicable

- IPC standards like IPC-610